Exercise #9 :  XRD basics

Chem 6614	Chemical Instrumentation

XRD Geek-o-rama

1) Match the three XRD spectra to the materials below:

Calcium silicate crystals__C___	  silica glass __B______	 glass ceramic ___A___

C

2a)  What is  in the Bragg equation,  2d sin   = n ?				     exiting beam

Angle of diffraction of x-ray beam 						

Incident beam

2b) What is d in the above equation ?

Diffracting material

3)  What is held constant in a classic Debye-Scherer XRD experiment  ?

a) d	b) d and  	c) d and 	d) 

4a) What is a typical wavelength for a typical X-ray e.g.   Cu-K photon ?

a) 1500 nm	b)  150 nm 	  c) 15 nm	d) 1.5 nm 	e) 0.15 nm	f) 0.015 nm

b) The physical size of the typical X-ray selected above is consistent with probing  typical distances between    _____atoms_________  (fill-in)

5) Why is the Debye-Scherer X-ray instrument referred to as a camera ?

Originally, the diffracted x-ray signals were recorded with film

6)  Circle all the significant information below obtainable via XRD

a) crystal structure	b) molecular structure	c) degree of devitrification	d) crystal phase

7a) Which is likely to provide more detailed information about crystal character ?

a) powder diffraction	b) single crystal diffraction

7b) Why  ?   powder form does not allow detailed determination of arrangement of atoms in cells