Exercise #1:  Solution Preparation Computations

Chem 6614	Chemical Instrumentation

Molar calculations

1)How many grams of Ni must be dissolved in 0.25 L of solution to create at 1 M Ni(II) solution ?   (at. Wt Ni = 58.68 g/mol)

2)How many grams of NiCl2*6H2O must be dissolved in a 0.25 L solution to create 1 M Ni(II) solution ? (at wt. Ni =58.68 g/mol ; MW NiCl2*6H2O = 237.64 g/mol)

3) You want to create a 0.2 M solution of Ni(II) in a 25 mL volumetric flask using the 1 M solution prepared above in problem . How would you go about making this solution ?

ppm calculations

4) What is the concentration in ppm of Ni in the 0.2 M solution you just

prepared ?

- 5 mL of the  0.2 M Ni solution are transferred to a 1 L flask and the  solution is diluted to the mark. What is the new Ni concentration in ppm ?

% w/v and v/v calculations

6) How many mL of concentrated HNO3  must be measured out to make 2 L of a 1%

v/v solution of HNO3 ?

7) How many grams of CuCl2*6H2O (MW= 170.44 g/mol) must be weighed out to prepare 500 mL of a  1% Cu(II) solution ? (at.mass Cu =63.54 g/mol)

Preparative Puzzle

Step 1) 42.61 g of CuCl2*2H2O are dissolved in 0.25 L.  Compute the solution’s Cu2+ concentration in M		(MW CuCl2*2H2O=170.44, at. Mass of Cu=63.54)

[Cu (M)]=				primary stock

Step 2)  5 mL of the above solution is diluted to 25 mL with 1% HNO3.  Compute the solution’s Cu2+ concentration in M and

[Cu (M)]=				molar reference standard

Step 3)  The solution made in step 2 is serially diluted first by transferring 1 mL to a 100 mL flask and diluting to the mark. A 20 mL sample of this intermediate solution is then transferred to a second 100 mL flask and diluted to the mark.  What is this `standard addition’  reference solution’s Cu2+ concentration in ppm  ?

[Cu(ppm)]=					Cu in standard addition ref. solution

Step 4) two standard additions (two 5 mL aliquots) of the standard addition solution above is diluted to 50 mL . What is the final standard addition concentration of Cu2+ (ppm)

[Cu(ppm)] =						concentration of std add solution #2

Conversions

9) A  1% v/v HNO3 is prepared by diluting 20 mL of 17 M concentrated HNO3 (MW=63 g/mol) to 2 L.

- what is the equivalent molarity of the HNO3 ?

- what is the equivalent ppt of the  HNO3 ?

- what is the ppm of N (atomic wt = 14.0 g/mol)

10) A 50 mL volume of 25 ppm solution of  Ni(II) (atomic wt= 58/68 g/mol)  is made

from NiCl2*6H2O (MW= 237.64 g/mol)

- what is the concentration of Ni(II) in μM units ?

b)  How many milligrams of of NiCl2*6H2O are contained in the 50 mL volume ?