Supplement #1: 	 Common Chemical Concentration Units  Summarized

Chem 6614	Chemical Instrumentation

Mole-based Concentration  Units

a) M= Molar= moles solute/Liters of solution 	=   	   grams solute	            # Liters

solute MW	            solution

b) mM = millimolar = mmoles solute/Liter of solution	=   milligrams solute            # Liters

solute MW		solution

c) μM  = micromolar = μmoles solute/Liter of solution  =   micrograms solute       # Liters

solute MW	          solution

Example:

0.0315 grams of HNO3  (MW= 63.0 g/mol) dissolved in 5 L

a)Molarity 			     =  0.0315 g           5 L								 							         63.0  g/mol

=0.0005 mol /5 L

=0.00001 M

= 1 x 10-5 M

b)Millimolar equivalent

1x10-5 M = 1*10-2 (* 10-3) M 	      = 1*10-2 mM		(recall:  10-3 = 1 milli unit = 1 m)

c)Micromolar equivalent

1x10-5 M = 1*101 *10-6                    = 10  μM			(recall: 10-6 = 1 micro unit = 1 μ )

- Similar conversions apply for the molal = moles solute/kg solvent  = m.  The molal is rarely used in analytical work, but is commonly used when colligative properties (freezing point depression, boiling point elevation and osmotic pressure) are considered.

Supplement #1: 	 Common Chemical Concentration Units  Summarized  (continued)		Chem 6614	Chemical Instrumentation

Weight-based Concentration Units

- ppt = parts-per-thousand =  grams solute solutes/Liters of solution

=  milligrams solute/ # milliliters of solution

- ppm = parts-per-million  =  milligrams solute/ Liters of  solution

=  micrograms/  milliliters of solute

c)      ppb = parts-per-billion     = micrograms solute/ Liters of solution

= nanograms solute/milliters of solution

Example

0.0500 grams of Cu metal are dissolved in 2.5 L of 1 M nitric acid

- ppt  Cu = 0.05 g Cu/2.5 Liters solution 			= 0.02 ppt

- ppm Cu =0.05 g x (103 mg/g) Cu/2.5 Liters solution     = 20 ppm

- ppb Cu  = 0.05 g x (106 μg/g) Cu/2.5 Liters solution     = 20000 ppb

Percent-based Concentration Units

- % solute (w/v) = 100 x (#grams solute/ #mL solution)   {w/v means weight/volume)

e)       % solute (v/v)   =100 x (#mL solute/# mL solution)        {v/v means volume/volume}

Example

5.00 grams Cu metal are dissolved in 2.5 L of 1 M nitric acid.

d)        % Cu (w/v) = 100 x  5.00g /(2.5 L *1000 mL)	    =  0.200 %  Cu w/v

L

5 mL of HNO3 is dissolved in 2 L of water.

- % HNO3  (v/v) =100x (5.00 mL/ [2 L  x 1000 mL]   = 0.25 % HNO3 v/v

L