Exercise #4: GC/HPLC  Practical Separations

Chem 6614	Chemical Instrumentation

4.1. General strategies

1) Pick a column (see attached table),  oven temperature schedule and GC detector that would be best to separate and analyze the % composition of mixture of alcohols with boiling points ranging from 120-180  oC so that the entire chromatogram takes no longer than 10 minutes to record.

2)The pyrenes (fused aromatic rings bp 300-500 C) collected from an arson site are eluted into ethanol. Suggest a method (GC or HPLC) to isolate and identify the various pyrene components. Suggest not only the technique but also a detector, column , flow rate and mobile phase composition.

3)The exhaust gas from a diesel generator needs to be analyzed for CO, CO2, unburnt hydrocarbons, and SO2. Suggest a GC method including a detector.

4.2. GC Nerd Factoids

4) What advantages do capillary columns have over packed columns ?

5) Name the various GC detectors that are:

- destructive of the sample:

- reliant on changes in conductivity

- universally applicable to all organics, insensitive but  cheap

- universally applicable to all organics  and ID’s compound but expensive

6)  List 3 ways to sharpen a GC peak

i)

ii)

iii)

7.) What does MAOT stand for ?______________________________________

A	B	C

7) List 3 ways to separate overlapping GC peaks

i)

ii)

iii)

8) What is the name of the common plumbing attachment

method shown to the right ?

A=___________________  B=_______________

What kind of fitting are we ?_________

C=__________________

3.3) HPLC Nerd Factoids

9) What does HPLC stand for ?______________________

10) What essentially takes the place of the thermal ramp in HPLC ?______________________

11) What is the specific name of the pump design in HPLC ?______________________

12) What levels out the pressure variation in the HPLC from the above pump ? _____________

13) If you use a polar carrier solvent, then you are operating in   reverse phase or	normal phase ?

14) What is a typical column length for HPLC ?

15) The following is written on an HPLC column’s metal `dog tag’: What does each term refer to ?

SOME COMMON STATIONARY (ACTIVE) GC PHASES (MAOT < 350 C)

stationary phase			commercial name		uses

Polymethyl silicone			OV-1, SE-30			general purpose

(methyl silicone)							(ROH, alkanes, alkenes, PCB,

Aromatics, drugs, steroids)

Poly(phenymethyl) siloxane		OV-17				aromatics, glycols, steroids

(50:50 methyl: phenyl)

Polyethylene glycol			carbowax 20M		acids, ethers, natural oils (fats)

Polyethylene glycol-

Terephthalic acid			OV-351			alcohols, fatty acids

Molecular sieve			Mol Sieve 5A,13X		permanent gases

(finely divided aluminosilicates, carbon	Poropak		(must be in packed column)

Similar cellular inorganics)

COLUMN TYPES = capillary, open 	 (>10’)		packed, closed (< 10’)

DETECTORS=TCD, PID, FID, ECD ,MS