EXAM #1: Chemical Instrumentation	6614		Spring 14

Your name:___________answers________________________

  - The Solution is… (10 pts total)   show work please !

a)  1 ppm w/v is defined as: (circle one answer)

i) 1 g/1 L 	  ii) 1 mg/ mL 		iii)    1 ug/1 L		iv) 1 mg/ 1 L

- You must prepare a 2.0 L aqueous  solution of  0.0029335  M Cu2+  from CuCl2*2H2O

(MW=170.44 g/mol). How many grams of CuCl2*2H2O must be weighed out to create

this solution ?  (3 pts)

2 L * 0.0029335 mol Cu/L * 1 mol CuCl2*2H2O/mol Cu * 170.44 g CuCl2*2H2O/mol =1 g

grams CuCl2*2H2O=___1_____

- You have a 1.0 L solution of 372.8 ppm Cu2+  (w/v) How many grams of CuCl2*2H2O were dissolved in the 1.0 L solution to reach this concentration ? The atomic mass of Cu =63.54 g/mol. The MW of CuCl2*2H2O =170.44 g/mol.

372.4 mg Cu/L= 0.3724 g Cu/L => 0.3728 g/63.54 g mol-1 = mol Cu = mol CuCl2*2H2O=> 0.00587 mol CuCl2*2H2O*170.44 g CuCl2*2H2O= g CuCl2*2H2O = 1.0 g

grams of CuCl2*2H2O=__1.0_

c)	A 5 mL volume  of concentrated acid HX  originally 0.5 M  in concentration is diluted to 1 L with distilled water to make a stock HX solution.  A 10 mL volume of this prepared stock is then delivered to a 500 mL volumetric flask to make an intermediate stock.  Finally, 0.5 mL of the intermediate stock are rediluted to a total of 25 mL in a 25 mL volumetric flask to make the final `standard’ HX .

What is the concentration of the final standard HX  in M  (micro moles/L).

Initial conc          1st dilution      2nd dilution      final dilution

Dilution = 0.5 x                5/1000   x   10/500      x    0. 5/25               = 1*10-6M =

1*10-6* (106 M/M)* M = 1.0 M

Final standard HX concentration = ____1.0______M

___/10

  - Fitting Rewards (12 pts total/ 4 pts each)

- A calibration of Absorbance (A) vs concentration (x (M))  of Cu2+ at  =813 nm yields the linear regression fit shown below. An unknown solution containing only Cu2+ has an A= 0.500 at 813 nm.  What is the concentration of Cu2+ in the unknown  in M (mol/L) ?

A = 1.02 x -0.01

0.5=1.02x-0.01

0.51/1.02=x=0.5

Unknown Cu2+ concentration = _______0.5____M

- A calibration of Cu standard additions (N) vs absorbance A at 352 nm using the AAS yields the fit below. 	Given that the standard addition volume is 5 mL and that one ( (1) standard addition volume of the unknown Cu is in each of the 25 mL volumes containing the calibration standards, what is the concentration of the unknown Cu solution ? Assume the reference concentration of the standard Cu is 10 ppm

A = 0.125*N + 0.0625

0= 0.125 Nu +0.0625

0.0625/0.125 = Nu=0.5

Nu*10= 0.5*10 ppm =5 ppm

Unknown Cu2+ concentration = _____5________ppm

  - What Every Atomic and UV-VIS Spectroscopy Geek should know…(40 pts)

- Acronym for the source of an AAS _____HCL ______________

- Durable older tranduricer that converts light to current in Lambda 4b UV-VIS __PMT  photomultiplier_______

- New school transduver that converts light to current in Lambda 25 UV-VIS: photodiode____

- What `gadget’ divides up the incoming source beam in a double beam UV-VIS into a reference and a source signal _____chopper______________ (hint: it looks like a propeller)

- ICP stands for…  ___________inductively coupled plasma____________

- UV-VIS cell material that provides the best window into the UV_quartz_______

- What is the `cell’ containing the sample analyzed in an AAS ?  _____burner head/flame___

- Name for the traditional  `circular’configuration of an ICP ? __Rowland circle____________

8)  Grating design in modern ICP __echelle________

9)  Name two advantages an AAS has over a UV-VIS for element analysis:

- ____________________no element interferences_(N2 problem gone)____

- ______________________higher sensitivity__________________

10) Atomic spectroscopy method of choice for small sample sizes: _electrothermal (graphite furnace

11) New-fangled grating used in Lambda 25 that simplifies the optical path: Holographic concave grating (focusing)

___/19

12) Name two advantages of ICP over AAS for element analysis:

- ________________no chemical interactions in flame_________________

- ________________simultaneous element analysis__________________

13) Item in an AAS that atomizes the sample so it can be burned: _nebulizer______

- Which are the double beam instruments below ? (circle all that apply, 2 pts)

Lambda 25 UV-VIS	Spec 20	  PE  Aanalyst AAS			ICP

- Russian sounding graphite sample holder for HGA _____Lvov platform_________

- Correct order for Electrothermal analysis:

  - flash- ash- dry   	b)dry-flash- ash      c)dry-ash-flash        d)flash-dry-ash

- For higher scan rate in a uv-vis:

a)increase slit width     b) decrease slit width   c) shorten wavelength range   d) increase PMT gain

19) Which technique is based on emission lines ? _____ICP___________________

20) Older style of ICP grating design: _________echellette_______________________

21) AAS and uv-vis both involve excitation of: (circle one)

a)vibrations		b)rotations 	    c)spin flips		d)electrons

22)	 Name for device in AAS and ICP that selects wavelength__monochromator____________

23) )The` cell` in an AAS is: the flame or slotted burner

24)	     What is the ~ temperature in an AAS flame ?   200 C       1200 C     2200 C        10,00 C

25)     What is the ~ temperature in an ICP flame        200 C       1200 C    2200 C         10000 C

26)      What is the gas excited in  both ICP and HCL ? ___argon_____________________

27)	What gas is excited to make uv light in the uv-vis spectrophotometer ?__D2_______

28)      What is the `source’ in the AAS experiment ? ___HCL lamp (or EDL)________________

29)         Which is more sensitive: ICP or UV-VIS ? ______ICP_______________

30)          Which technique uses the least sample ?

a) HGA (electrothermal)	b) ICP		c) AAS-flame		d) UV-VIS

__/18

33)     The following plot is obtained for a calibration of an AAS standard addition of Ni with an   unknown. What’s wrong and what  error is likely to have occurred during the solution preparations ?	(3 pts)

Forgot to put in unknown aliquot		A

which is why  A=0 at N=0 std adds of reference

Ni

0          1        2       3       4

N= # std additions of 10 ppm Ni

34)    For a mixture of 5 unknowns how many calibration plots are necessary to solve for the unknown concentrations using a UV-VIS analysis ? ____52 =25__________________  (2 pts)

____/16

  - IR BASICS (38 pts/2 pts each, except where noted)

1.)	Range of the mid-IR region:_____200-4000 cm-1____(must show units)____________

2.) The diagnostic region measures what basic vibrational motion ?__stretch___________

3.) The fingerprint region measures what basic vibrational motion ?__bends_____________

4.) If you have a 10 atom, non-linear molecule, how many possible vibrations are possible?

_____3*10-6=24__________

5.)  Would you expect O2 to have active IR bands ? 		Yes		No   1 pt

6.) What is the name of the key `trick’ in FFT that offsets light to produce interferograms ?

____michelson-morley interferometer_	(hint: two Physics geeks from the early days)

7.) Name two advantages of FFT

i)Faster, fellgett noise reduction,

ii)	 higher light throughput (Jacquinot), better resolution

9.)The basic IR effect involves __vibrational__________________ transition in the ____ground___________electronic state   (1 pt each)

10.)  The abbreviation vs stands for: ____very strong_______________________

____/19

1.4.  IR Basics (continued)

11.)  The correct order of analysis typical for a dispersive IR spectrophotometer is:

a)   source slitsample and reference chopper monochromator detector

b)  source slit chopper sample or reference monochromator detector

c) chopper source sample or reference monochromator detector

d) monochromator source chopper sample or reference detector

12.)  A strong, broad band centered near 3500 cm-1 is what molecular motion ?____O-H stretch__

13)  A sharp set of strong multiplets just below 3000 cm-1 is what molecular motion ? C-C-H stretch

14.)  Key frequency indicative of a C=O (carbonyl) stretch: _____~1700__________ cm-1

15.)  A medium strong doublet near 1380 cm-1` is not accompanied with a similar band near 1430 cm-1 . The most likely compound consistent with this below is: (circle your answer)

no CH2 or C H implied by absence of 1380…all CH3

16)  what device in a dispersive IR is `moved’ to balance the reference and sample sides

of the instrument ?

a) monochromator   b) chopper	c) beam splitter	d) attenuator   e)floating mirror on air

bearing

17.) What device in a dispersive IR is responsible for moving the device in question 16 ?

a) beam splitter`	b) chopper	c) attenuator	d) synchronous motor	    e) tuning rectifier

18.)  What does FFT stand for ? ___Fast Fourier Transform______________________________

19.)  What physically scans in an FFT ?

a) beam splitter	b) moving mirror	c) monochromator	d) laser interferogram

____/18